# RITES DPR Appraisal Tool – Check Summary

The following checks are required as part of the DPR appraisal process. Each point describes the check area, what is being verified, and the required inputs.

- Alignment Data Validation (Alignment): Check whether the uploaded alignment polyline matches the DPR alignment drawings / GIS file, requiring DPR alignment file, CAD/GIS polyline, KML/Shapefile as input.

- Defence Area / Mining Area / Other No-Go / Restricted Area (Alignment – Key Points): Check whether the alignment passes through these areas, requiring API-BiSAG/GIS base map as input.

- Forest / Wildlife Area (Alignment – Key Points): Check whether the alignment passes through these areas, requiring API-BiSAG/GIS base map; mitigation measure to be included as input.

- Obligatory Points (Alignment – Key Points): Define obligatory points like stations and economic nodes, requiring API-BiSAG/GIS base map as input.

- Yard Congestion / Habitation / Land Cost (Alignment – Key Points): Check for congestion or space constraints at yards; land cost above ₹2 cr/acre, requiring Land cost data (threshold: >2 cr/acre) as input.

- Coordinate Validation (Alignment): Check whether station coordinates and alignment coordinates are correctly geo-referenced, requiring Station coordinates, GIS base map as input.

- Corridor Length Check (Alignment): Compare GIS-derived alignment length with DPR-reported length, requiring DPR length statement, GIS alignment as input.

- Curve Radius Compliance (Technical Feature of Alignment): Check if curves comply with minimum radius requirements for speeds of 160 km/hr and 130 kmph, requiring 1.3 Degree (160 kmph), 2.2 Degree (130 kmph); for new line projects: 1 Degree as input.

- Gradient Compliance (Technical Feature of Alignment): Check whether vertical gradients are within permissible limits, requiring Ruling gradient 1:100 (compensated); approach to river 1:150; station/yards 1:1200 or maximum 400 as input.

- Design Speed (Track): Check the design speed provided in DPR, requiring Design speed of 160 kmph as input.

- Route Km (RKm) (Track): Check the Route Km in line with corridor length findings, requiring Finalized alignment as input.

- Track Km (TKm) (Track): Compare Route Km and Track Km to ensure consistency, requiring TKm < 1.2 × RKm (doubling project); TKm < 2.4 × RKm (3rd and 4th line) as input.

- Design Axle Load (Track): Check whether loading standards are within permissible levels, requiring 25 tons axle load standard as input.

- Minimum Formation Width (Track): Check the minimum formation width as per standards, requiring 7.85 m for single line; 13.15 m for double line as input.

- Track Centre Spacing (Track): Check the minimum track centre spacing between lines, requiring Minimum 5.3 m (for doubling); minimum 7.8 m for 3rd line and 5.3 m for 4th line from 3rd line as input.

- Design Loading (Bridges): Verify loading standards for bridge sub-structure and super-structure, requiring Sub-structure: 25T loading; Super-structure: 25T loading as input.

- Sectioning and Paralleling Post (SP) (Electrification): Check the number of SPs per Route Km, requiring One SP per 50 km of route length as input.

- Sub-Sectioning and Paralleling Post (SSP) (Electrification): Check the number of SSPs per Route Km, requiring Two SSPs per 50 km of route length as input.

- Traction Sub Station (TSS) (Electrification): Check the number of TSSs per Route Km, requiring One TSS at every 50 km of route length as input.

- Tower Wagon / Car & Shed (Electrification): Check the number of tower wagons/cars and sheds provided, requiring One per 50 Route Km or 100 Track Km as input.

- SCADA (Supervisory Control and Data Acquisition) (Electrification): Check Remote Terminal Unit count to determine if a new SCADA system is required, requiring If Remote Terminal Units > 50, a new SCADA system is required as input.

- Provision of EI (Electronic Interlocking) (Signalling): Check the standard of Electronic Interlocking provided against required speed, requiring Standard-IV for 160 kmph (Standard-I: up to 50 kmph, Standard-II: up to 100 kmph, Standard-III: up to 130 kmph) as input.

- System of Working (Signalling): Check the appropriate system of working selected for the route type, requiring HDN/HUN/Sub-Urban/3rd and 4th line routes require Automatic Signalling; Absolute or Automatic Block System as applicable as input.

- Track Circuit (Signalling): Verify track circuit type for HDN/GQ/GD and tunnel sections, requiring Multi-Sectional Digital Axle Counters (MSDAC) as input.

- Automatic Train Protection System (Signalling): Check whether Kavach is provided for all new projects, requiring Kavach system for all new projects as input.

- Remote Diagnostic & Predictive Maintenance System (RDPMS) (Signalling): Verify RDPMS provision for all automatic routes, requiring Yes/No confirmation for all automatic routes as input.

- Line Capacity (Traffic): Assess existing and projected line capacity with and without maintenance block under automatic signalling, requiring Projected line capacity should be between 70% and 100% as input.

- Catchment Coverage (Traffic): Assess population within buffer zones of 5 km, 10 km, and 25 km around the corridor, requiring Population assessment in buffer zone using BiSAG (Yes/No) as input.

- Stakeholder Consultations (Traffic): Check whether consultations have been conducted with all relevant ministries, departments, and industry stakeholders, requiring Ministry of Coal, Steel, Heavy Industries, Home Affairs, Environment, Shipping, Tourism, Telecom, Forest Dept, District Administration, State Electricity Dept, Industry Associations, Steel Plants, Thermal Power Plants, Cement/Fertilizer/Food Corporation plants, and others as input.

- Tourist Destinations (Traffic): List tourist destinations using BiSAG and assess existing footfalls, requiring Yes/No confirmation via BiSAG as input.

- Interchange Potential (Traffic): Check connectivity with railway stations, metro, bus terminals, airports, major roads, and multimodal logistics hubs, requiring Transport network layers as input.

- First / Last Mile Connectivity (Traffic): Assess existing road networks and identify requirements for widening, substantiated with photographs, requiring Yes/No confirmation for assessment of roads and economic nodes around the project section as input.

- Total Land – Private / Government / Forest Bifurcation (Land Requirement & Cost): Check whether land has been bifurcated into private, government, and forest categories, requiring Yes/No as input.

- Total Land – State-wise Requirement (Land Requirement & Cost): Check whether state-wise land requirement has been provided, requiring Yes/No as input.

- Rural Factor (Land Cost): Verify the rural land cost factor is within permissible limit, requiring Factor value should be less than 2 as input.

- Urban Factor (Land Cost): Verify the urban land cost factor is within permissible limit, requiring Factor value should be less than 1 as input.

- Administration Cost (Land Cost): Verify administration cost percentage, requiring Should be greater than 3% as input.

- Contingencies (Land Cost): Verify contingency provision percentage, requiring Should be greater than 5% as input.

- Interest (if more than 1 year) (Land Cost): Check interest provision for projects exceeding one year, requiring Factor value should be greater than 1 as input.

- FIRR (Financial Internal Rate of Return) (Financial Analysis): Check the return on project against the minimum threshold, requiring Return on project should not be less than 10% as input.

- EIRR (Economic Internal Rate of Return / Equity IRR) (Financial Analysis): Check the equity IRR against the minimum threshold, requiring Equity IRR should not be less than 18% as input.

- Total Project Cost – New Lines (Financial Analysis): Check the total project cost per Route Km for new line projects, requiring Cost should be between ₹25 cr/RKm and ₹28 cr/RKm as input.

- Total Project Cost – Doubling (Financial Analysis): Check the total project cost per Route Km for doubling projects, requiring Cost should be between ₹15 cr/RKm and ₹18 cr/RKm as input.

- Total Project Cost – Gauge Conversion (Financial Analysis): Check the total project cost per Route Km for gauge conversion projects, requiring Cost should be between ₹7 cr/RKm and ₹8 cr/RKm as input.